SWAPKEYS

CAPS_LOCK/CTRL key swapper.

SWAPKEYS swaps the functionality of the CAPS_LOCK key and the left CONTROL

key. It also remaps what the keyboard thinks is CAPS-PAUSE into

CTRL-BREAK and what the it thinks is CTRL-BREAK into PAUSE.

Usage:

To install SWAPKEYS, type

C>swapkeys

at the DOS prompt.  That's it.

Behavior:

SWAPKEYS will install itself in one of two ways.  It will either

hook Interrupt 9H (the keyboard) or Interrupt 15H (the miscellaneous

services). The preferred method is to use INT 15H for two reasons:

1) it uses less memory

2) Function 4FH, the function it traps, was designed for

this type of thing.

Unfortunately, not every BIOS supports INT 15H, function 4FH. If

SWAPKEYS is going to use INT15H, it sign-on display will look like:

SWAPKEYS   Version 1.0 (MS-DOS)

(c) Copyright 1991 Latham King

SWAPKEYS has been installed.  Hooking Int 15H.

If on the other hand, it must use INT 9H, SWAPKEYS's sign-on will

display	will be:

SWAPKEYS   Version 1.0 (MS-DOS)

(c) Copyright 1991 Latham King

SWAPKEYS has been installed.  Hooking Int 9H.

Memory Usage:

Using the Int 15H Interrupt Service Routine:	336 bytes

Using the Int 9H Interrupt Service Routine:	576 bytes

Compatibility:

This TSR has been tested and verified with the following ROM BIOS's:

Phoenix 80486 ROM BIOS PLUS Version 0.10 G10

Gateway 2000 486

Premium 386 BIOS v1.3

AST Premium 386

American Megatrends 386-BIOS (c) 1989

IBM PS/2 Model 50

IBM PS/2 Model L40SX (80386 SX)

Toshiba BIOS (c) 1990 in the:

T5100

T3200SX

T3200SXC

Compaq BIOS dated 1988 in the:

Compaq 386/20 (80386)

Compaq Portable III (80286)

Phoenix ROM BIOS Version 3.03 in the AST Premium 286

PC's Limited 286 ROM BIOS

(c) 1986 Dell Computer Corp. &

(c) 1986 AMI

Phoenix 80286 ROM BIOS PLUS Version 3.10 22

Nec Powermate 2

This is SHAREWARE software.  So, if you find yourself using it, please

pay for it by sending $20 US (or what ever you think this software is worth)

to me at:

Latham King

P.O. Box 957

Darien, CT  06820

USA

Comments and any other random musings can be direct to the above

address or to CompuServe account 70313,662.

Thank you and I hope this software proves useful.

- Latham King